| Project | FusionDrive1MB | Confidentiality |  |
| --- | --- | --- | --- |
| Version | 0.1 | Page |  |

BootROM Software Technology Solutions

---FusionDrive1MB

- Document Change History

| Release Version | Date | Change Description |
| --- | --- | --- |
| 0.1 | 2024/09/05 | Initial release |

Document Change History	1

1.	Introduction	3

1.1.	System Overview	3

Introduction

System Overview

References

Terms and Abbreviations

Restrictive Constraints of Design

| Class | Restrictive |
| --- | --- |
| CSE | shall complies with HIS-SHE specification 1.1 |

Software architecture

Architecture

System Resource Design

CPU Resource

RAM & Flash Space

| Index | Fusion-Drive-1M | Fusion-Drive-256K |
| --- | --- | --- |
| 1 | Program-0 : 512KB Program-1 : 512KB Data flash: 64KB | Program-0 : 256KB Program-1 : N/A Data flash: 64KB |
| 2 | Co-work with CSE module | No CSE module |
| 3 | Sector size: Program flash: 1KB Data flash: 512Bytes | Sector size: Program flash: 512Bytes Data flash: 512Bytes |
| 4 | Boot-rom size: 24KB | 6KB |
| 5 | Support RWW OTA | Not support RWW OTA |

Interrupt

| Device | Interrupt Number | Description |
| --- | --- | --- |
| USART/LIN |  | Use to Burn /Download Firmware、Update KEY |
| Timer |  |  |
| SPI |  | Use to Burn /Download Firmware、Update KEY |

Software Design

Function

Command

Firmware Info

| Field Name | Range | Length(Byte) |
| --- | --- | --- |
| Burn Addr | Bank0/Bank1 Flash Address | 4 |
| Total Length | 0x1~(0x100000 – 0x200) | 4 |
| version | User define | 4 |
| is encrypt | 0:no need encrypt 1:need encrypt | 1 |
| decrypted_address | RAM Space Address | 4 |
| CMAC Value | CMAC Value for Firmware protected Area | 16 |
| KEY ID used for encrypt | KEY1~KEY20 MASTER_ECU_KEY | 1 |

Command List

| Command | Code | Description |
| --- | --- | --- |
| Get Support Command[SEC] | 0x0 | Get the version of the bootloader and the commands supported |
| Get Version(LIN Only?) [SEC] | 0x1 | Get the version of the bootloader |
| Get Group(LIN Only?) [SEC] | 0x2 | Get the group which the chip belongs to |
| Speed Change(LIN Only?) [SEC] | 0x3 | The speed command allows the baud rate for CAN run-time to be changed. |
| Program Data(LIN Only?) | 0x4 | Data content with checksum |
| PRODUCT ID(CAN Only?) | 0x6 | Get product id |
| Read | 0x11 | Read no more than 256 bytes data from specified address |
| Jump | 0x21 | Jump to an address to execute the codes there |
| Program Prepare | 0x30 | Prepare to program, send some firmware overview |
| Program | 0x31 | Program no more than 256 bytes data from a specified address |
| Erase(UART Only?) | 0x43 | Erase one or more pages of the main flash |
| Extend Erase | UART:0x44 LIN:0x14 | Erase one or more pages of the main flash using 2 bytes addressing information |
| Enable Erase/Program Protection | UART:0x63 LIN:0x0B | Enable some pages under the protection from erasing or programming |
| Disable Erase/Program Protection | UART:0x73 LIN:0x1B | Disable the protection from erasing or programming on the whole main flash |
| Enable Security Protection | UART:0x82 LIN:0x0D | Enable the security protection |
| Disable Security Protection[SEC] | UART:0x92 LIN:0x1D | Disable the security protection |
| Update KEY | 0xA1 | Update CSE Key |
| Delete All Key | 0xA2 | Delete All Key |
| Get Firmware Info | 0xA3 | Get Firmware info |

Firmware Return Info

Burn Firmware

The firmware flashing function can be used to flash the firmware through BootROM. Users can either flash a firmware program or a multi-segment firmware program, and the user is responsible for the management of the relationship between the multi-segment firmware programs.

BootROM is only responsible for the management of the main entrance firmware program (verification, decryption, execution, etc.)

Download Firmware

The firmware download function can be used for users to download programs to memory via BootROM, which can be used for temporary debugging needs.

Update KEY

The key update feature can be used to update the user key, MASTER_ECU_KEY, BOOT_MAC_KEY via BooROM.

Burn Data

The flashing function can be used to burn the data required by the firmware to Flash.

Data Structure

firmware_info_t

typedef struct firmware_info {

uint32_t address;           /* firmware start address */

uint32_t length;            /* firmware length */

uint32_t version;           /* firmware version */

uint8_t  type;              /* firmware type */

uint8_t  is_encrypted;      /* is need to encrypt? */

uint8_t  encrypt_key_id;    /* must be update key before burn */

uint32_t decrypted_address; /* decrypted memory address */

uint8_t  cmac[16];          /* cmac value use BOOT_CMAC_KEY */

} firmware_info_t

Update_key_info_t

typedef struct {

uint8_t auth_keyid;

uint8_t auth_key[KEY_LENGTH];

uint8_t new_keyid;

uint8_t new_key[KEY_LENGTH];

} update_key_info_t;

Software Flow

Initialize & Startup

OTA & ISP Update

ISP Common Flow Chart

USART

LIN

SPI

Get Command

UART

PC Flow

Boot Flow

LIN

PC Flow

Boot Flow

Get BootROM Version Command[LIN Only]

LIN

PC Flow

Boot Flow

Get Group Command[LIN Only]

LIN

PC Flow

Boot Flow

Speed Command[LIN Only]

LIN

PC Flow

Boot Flow

Read Command

UART

PC Flow

Boot Flow

LIN

PC Flow

Boot Flow

Jump Command

UART

PC Flow

Boot Flow

LIN

PC Flow

Boot Flow

Prepare Program Command[New]

UART

PC Flow

Boot Flow

LIN

Same as UART

Program /Program Data Command[Modify]

UART

PC Flow

Boot Flow

LIN

PC Flow

Boot Flow

Program Firmware Info

Erase Command[Modify]

UART

PC Flow

Boot Flow

Extend Erase Command

UART

PC Flow

Boot Flow

LIN

PC Flow

Boot Flow

Enable Erase/Program Protection Command

UART

PC Flow

Boot Flow

LIN

PC Flow

Boot Flow

Disable Erase/Program Protection Command

UART

PC Flow

Boot Flow

LIN

PC Flow

Boot Flow

Enable Security Protection Command

UART

PC Flow

Boot Flow

LIN

PC Flow

Boot Flow

Disable Security Protection Command

UART

PC Flow

Boot Flow

LIN

PC Flow

Boot Flow

Update Key Command[New]

UART

PC Flow

Boot Flow

LIN

Same as UART

Delete All Key Command[New]

UART

PC Flow

Boot Flow

LIN

Same as UART

Get Firmware Information Command[New]

UART

PC Flow

Boot Flow

Execute Firmware Code

Update KEY

Update KEY

Update BOOT_MAC

Flash Write

Normal Write

Fast Write

Option Byte Write